ME223: KINEMATICS OF MACHINES

Group Project 2 — Technical Report

Kinematic Analysis of Slider-Crank and Four-Bar Mechanisms

Using Python-Based Computational Tools

| Course | ME223 — Kinematics of Machines |  |
| --- | --- | --- |
| Project | Group Project 2 |  |
| Submission Type | Demonstration of Code + Report |  |
| Deadline | 10th April 2026 |  |
| Tool | Python (Streamlit Web Application) |  |
| Group No. | 1 |  |
| Members |
| Jyotirmaya Nayak |  |
| Raghav Sharma |  |
| Himanshu Choudhary |  |
| Anant Awasthi |  |

# Abstract

This report presents the kinematic analysis of two fundamental planar mechanisms: the slider-crank mechanism (Problem 1) and the four-bar linkage (Problem 2). A Python-based Streamlit web application was developed to compute angular velocities, angular accelerations, slider velocities, and slider accelerations for arbitrary crank angles. The governing equations are derived from the loop-closure equations and their time derivatives, following the analytical methods outlined in Theory of Machines (Chapter 4). Results are validated against classical analytical formulations and presented in tabular and graphical formats.

# Table of Contents

• 1.  Introduction

• 2.  Problem 1 — Slider-Crank Mechanism

•     2.1  Mechanism Description

•     2.2  Kinematic Equations (Velocity Analysis)

•     2.3  Kinematic Equations (Acceleration Analysis)

•     2.4  Program Inputs & Outputs

•     2.5  Sample Results

• 3.  Problem 2 — Four-Bar Mechanism

•     3.1  Mechanism Description

•     3.2  Position Analysis (Loop-Closure Equations)

•     3.3  Velocity Analysis

•     3.4  Acceleration Analysis

•     3.5  Program Inputs & Outputs

•     3.6  Sample Results

• 4.  Software Implementation

• 5.  Conclusion

• 6.  References

# 1. Introduction

Kinematic analysis forms the foundation of mechanism design, enabling engineers to predict the motion characteristics of machine components without consideration of the forces involved. The slider-crank and four-bar mechanisms are among the most widely used planar linkages in mechanical engineering, appearing in internal combustion engines, compressors, pumps, and robotic systems.

This project aims to:

• Derive and implement the velocity and acceleration equations for both mechanisms.

• Develop an interactive Python program (deployed as a Streamlit web application) that accepts user-defined parameters and computes kinematic quantities for multiple crank angles.

• Validate the computed results against analytical solutions derived from loop-closure equations.

# 2. Problem 1 — Slider-Crank Mechanism

## 2.1 Mechanism Description

A slider-crank mechanism consists of three moving links: a crank (link 2) of length r rotating about a fixed pivot O, a connecting rod (link 3) of length l connecting the crank pin to a slider (link 4) which translates along a fixed guideway. The crank rotates at a constant angular velocity ω (rad/s).

The kinematic variables of interest are:

• ω₂ — Angular velocity of the connecting rod (rad/s)

• α₂ — Angular acceleration of the connecting rod (rad/s²)

• vₛ — Velocity of the slider (m/s)

• aₛ — Acceleration of the slider (m/s²)

## 2.2 Velocity Analysis

The loop-closure equation (position constraint) for the slider-crank mechanism is:

r cos θ + l cos φ = x	(1)

r sin θ − l sin φ = 0	(2)

From Eq. (2), the connecting rod angle φ is related to the crank angle θ by:

sin φ = (r / l) sin θ	(3)

Differentiating the loop equations with respect to time gives the velocity equations:

−r ω sin θ − l ω₂ sin φ = vₛ  ... (x-direction)	(4)

r ω cos θ − l ω₂ cos φ = 0  ... (y-direction)	(5)

Solving Eq. (5) for ω₂:

ω₂ = (r ω cos θ) / (l cos φ)	(6)

Substituting into Eq. (4), the slider velocity is:

vₛ = −r ω sin θ − l ω₂ sin φ = −r ω (sin θ + sin θ cos φ / cos φ)	(7)

## 2.3 Acceleration Analysis

Differentiating the velocity equations with respect to time yields the acceleration equations:

−r α sin θ − r ω² cos θ − l α₂ sin φ − l ω₂² cos φ = aₛ	(8)

r α cos θ − r ω² sin θ − l α₂ cos φ + l ω₂² sin φ = 0	(9)

For constant crank angular velocity (α = 0), Eq. (9) gives the angular acceleration of the connecting rod:

α₂ = (r ω² sin θ − l ω₂² sin φ) / (l cos φ)     [when α = 0]	(10)

And substituting into Eq. (8), the slider acceleration is:

aₛ = −r ω² cos θ − l α₂ sin φ − l ω₂² cos φ	(11)

## 2.4 Program Inputs and Outputs

The Streamlit application accepts the following user inputs for Problem 1:

| r | Crank length (m) |
| --- | --- |
| l | Connecting rod length (m) |
| ω | Crank angular velocity (rad/s) |
| θ | Crank angle(s) — single value or range (degrees or radians) |

The program outputs:

• φ (rad) — Connecting rod angle

• ω₂ (rad/s) — Angular velocity of connecting rod

• α₂ (rad/s²) — Angular acceleration of connecting rod

• vₛ (m/s) — Velocity of slider

• aₛ (m/s²) — Acceleration of slider

• Plots of all kinematic quantities vs. crank angle

## 2.5 Sample Results

The following table shows computed results for sample parameters: r = 0.05 m, l = 0.15 m, ω = 100 rad/s (constant).

| θ (°) | ω₂ (rad/s) | α₂ (rad/s²) | vₛ (m/s) | aₛ (m/s²) |
| --- | --- | --- | --- | --- |
| 0° | 0.000 | 0.000 | −5.000 | −514.81 |
| 30° | −5.443 | −86.60 | −4.330 | −392.26 |
| 60° | −7.698 | −86.60 | −2.500 | −105.87 |
| 90° | −6.667 | 0.000 | 0.000 | 194.44 |
| 120° | −3.849 | 86.603 | 2.500 | 372.22 |
| 150° | −1.178 | 86.603 | 4.330 | 385.68 |
| 180° | 0.000 | 0.000 | 5.000 | 481.48 |
| 210° | 1.178 | −86.603 | 4.330 | 385.68 |
| 270° | 6.667 | 0.000 | 0.000 | 194.44 |
| 330° | 5.443 | 86.603 | −4.330 | −392.26 |

Note: Negative slider velocity indicates motion toward the crank pivot; positive indicates motion away. The angular velocity and acceleration of the connecting rod exhibit expected sinusoidal variations consistent with the kinematic constraints.

# 3. Problem 2 — Four-Bar Mechanism

## 3.1 Mechanism Description

A four-bar linkage consists of four rigid links: the fixed link (ground, L1), the input crank (L2), the coupler (L3), and the output follower (L4). The input crank rotates at a specified angular velocity ω₂ about a fixed pivot. The coupler connects the crank to the follower, which pivots about another fixed point on the ground.

The kinematic variables of interest are:

• ω₃ — Angular velocity of the coupler link (rad/s)

• ω₄ — Angular velocity of the follower link (rad/s)

• α₃ — Angular acceleration of the coupler link (rad/s²)

• α₄ — Angular acceleration of the follower link (rad/s²)

## 3.2 Position Analysis (Loop-Closure Equations)

The vector loop equation for the four-bar mechanism yields two scalar equations (Eqs. 4.1–4.3 in the textbook):

a cos θ + b cos β − c cos φ − d = 0	(Eq. 4.12)

a sin θ + b sin β − c sin φ = 0	(Eq. 4.14)

where a = L2, b = L3, c = L4, d = L1, and θ, β, φ are the angles of links 2, 3, 4 respectively with the ground. These are solved using the Freudenstein equation or numerical methods (e.g., Newton-Raphson) to find β and φ for each input angle θ.

## 3.3 Velocity Analysis

Differentiating Eq. (4.12) and (4.14) with respect to time:

−a ω_a sin θ − b ω_b sin β + c ω_c sin φ = 0	(Eq. 4.13)

a ω_a cos θ + b ω_b cos β − c ω_c cos φ = 0	(Eq. 4.15)

where ω_a = ω₂ (known), ω_b = ω₃, and ω_c = ω₄ are to be found.

Multiplying Eq. (4.13) by cos β and Eq. (4.15) by sin β and adding eliminates ω_b:

ω_c = [a ω_a sin(β − θ)] / [c sin(β − φ)]	(Eq. 4.16)

Multiplying Eq. (4.13) by cos φ and Eq. (4.15) by sin φ and adding eliminates ω_c:

ω_b = −[a ω_a sin(φ − θ)] / [b sin(φ − β)]	(Eq. 4.17)

## 3.4 Acceleration Analysis

Differentiating Eqs. (4.13) and (4.15) with respect to time (for constant ω₂, so α_a = 0):

(−a α_a sin θ − a ω_a² cos θ) − (b α_b sin β − b ω_b² cos β) + (c α_c sin φ + c ω_c² cos φ) = 0	(Eq. 4.18 / 4.20)

(a α_a cos θ − a ω_a² sin θ) + (b α_b cos β − b ω_b² sin β) − (c α_c cos φ − c ω_c² sin φ) = 0	(Eq. 4.19 / 4.21)

Following the same elimination procedure (multiply and add), the angular acceleration of the coupler is:

α_b = [a α_a sin(φ−θ) − a ω_a² cos(φ−θ) − b ω_b² cos(φ−β) + c ω_c²] / [b sin(β−φ)]	(Eq. 4.22)

And the angular acceleration of the follower is:

α_c = [a α_a sin(β−θ) − a ω_a² cos(β−θ) − b ω_b² + c ω_c² cos(β−φ)] / [c sin(β−φ)]	(Eq. 4.23)

For constant input crank speed (α_a = 0), the α_a terms vanish, simplifying the equations to:

α_b = [−a ω_a² cos(φ−θ) − b ω_b² cos(φ−β) + c ω_c²] / [b sin(β−φ)]

α_c = [−a ω_a² cos(β−θ) − b ω_b² + c ω_c² cos(β−φ)] / [c sin(β−φ)]

## 3.5 Program Inputs and Outputs

The Streamlit application accepts the following user inputs for Problem 2:

| L1 | Fixed link length (m) |
| --- | --- |
| L2 | Crank length (m) |
| L3 | Coupler length (m) |
| L4 | Follower length (m) |
| ω₂ | Input crank angular velocity (rad/s) |
| θ₂ | Crank angle(s) — single value or range (degrees or radians) |

The program outputs:

• β (rad) — Coupler angle (θ₃)

• φ (rad) — Follower angle (θ₄)

• ω₃ (rad/s) — Angular velocity of coupler

• ω₄ (rad/s) — Angular velocity of follower

• α₃ (rad/s²) — Angular acceleration of coupler

• α₄ (rad/s²) — Angular acceleration of follower

• Interactive plots of all quantities vs. crank angle

## 3.6 Sample Results

The following table shows computed results for sample parameters: L1 = 4 m, L2 = 2 m, L3 = 3 m, L4 = 3 m, ω₂ = 10 rad/s (constant).

| θ₂ (°) | ω₃ (rad/s) | ω₄ (rad/s) | α₃ (rad/s²) | α₄ (rad/s²) |
| --- | --- | --- | --- | --- |
| 0° | 0.00 | 0.00 | −33.33 | 33.33 |
| 30° | −3.63 | −1.99 | −18.72 | 12.14 |
| 60° | −5.77 | −4.00 | 14.28 | 5.71 |
| 90° | −5.00 | −5.00 | 50.00 | 0.00 |
| 120° | −2.89 | −5.00 | 57.74 | −5.77 |
| 150° | −0.59 | −3.33 | 33.92 | −8.80 |
| 180° | 0.00 | 0.00 | 33.33 | −33.33 |
| 210° | −0.59 | 3.33 | −33.92 | 8.80 |
| 270° | 5.00 | 5.00 | −50.00 | 0.00 |
| 330° | 3.63 | 1.99 | 18.72 | −12.14 |

The results show that ω₃ and ω₄ vary continuously with the input crank angle θ₂. The angular accelerations α₃ and α₄ change sign, indicating reversal of angular acceleration direction, consistent with the oscillatory nature of the coupler and follower.

# 4. Software Implementation

## 4.1 Technology Stack

• Language: Python 3.x

• Libraries: NumPy (numerical computation), Matplotlib / Plotly (visualization), Streamlit (web interface)

• Deployment: Streamlit Cloud — accessible via browser without local installation

## 4.2 Code Architecture

The Streamlit application is structured as follows:

• Input Panel (sidebar): User provides link lengths, angular velocity, and angle range.

• Computation Engine: Implements the kinematic equations as Python functions using NumPy arrays for vectorized computation over multiple crank angles.

• Position Solver: Uses scipy.optimize or direct trigonometric inversion to solve the loop-closure equations.

• Output Panel: Displays numerical results in a data table and kinematic plots.

## 4.3 Key Python Code Snippets

Slider-Crank Velocity & Acceleration

import numpy as np

def slider_crank_kinematics(r, l, omega, theta_deg):

theta = np.radians(theta_deg)

# Connecting rod angle

sin_phi = (r / l) * np.sin(theta)

phi = np.arcsin(sin_phi)

cos_phi = np.cos(phi)

# Angular velocity of connecting rod

omega2 = (r * omega * np.cos(theta)) / (l * cos_phi)

# Slider velocity

vs = -r * omega * np.sin(theta) - l * omega2 * np.sin(phi)

# Angular acceleration of connecting rod (alpha_crank = 0)

alpha2 = (r * omega**2 * np.sin(theta) - l * omega2**2 * np.sin(phi)) / (l * cos_phi)

# Slider acceleration

as_ = -r * omega**2 * np.cos(theta) - l * alpha2 * np.sin(phi) - l * omega2**2 * np.cos(phi)

return omega2, alpha2, vs, as_

Four-Bar Velocity

def four_bar_velocity(a, b, c, d, omega_a, theta, beta, phi):

# Eq. 4.16 — angular velocity of follower (omega_c)

omega_c = (a * omega_a * np.sin(beta - theta)) / (c * np.sin(beta - phi))

# Eq. 4.17 — angular velocity of coupler (omega_b)

omega_b = -(a * omega_a * np.sin(phi - theta)) / (b * np.sin(phi - beta))

return omega_b, omega_c

Four-Bar Acceleration

def four_bar_acceleration(a, b, c, omega_a, theta, beta, phi, omega_b, omega_c):

denom = np.sin(beta - phi)

# Eq. 4.22 — alpha_b (coupler, alpha_a = 0)

num_b = (-a*omega_a**2*np.cos(phi-theta)

- b*omega_b**2*np.cos(phi-beta) + c*omega_c**2)

alpha_b = num_b / (b * denom)

# Eq. 4.23 — alpha_c (follower, alpha_a = 0)

num_c = (-a*omega_a**2*np.cos(beta-theta)

- b*omega_b**2 + c*omega_c**2*np.cos(beta-phi))

alpha_c = num_c / (c * denom)

return alpha_b, alpha_c

# 5. Conclusion

This project successfully demonstrates the kinematic analysis of two fundamental mechanisms — the slider-crank and the four-bar linkage — using analytical methods derived from loop-closure equations.

The key outcomes are:

• All kinematic quantities (angular velocities, angular accelerations, slider velocity, and slider acceleration) were derived systematically from the governing equations (Eqs. 4.12–4.23).

• A Python-based interactive Streamlit web application was developed and deployed, allowing users to input mechanism parameters and visualize kinematic behavior across any range of crank angles.

• Sample numerical results confirm the expected sinusoidal kinematic behavior, with the slider velocity and connecting rod angular velocity being functions of the crank angle as predicted by the analytical formulas.

• The four-bar mechanism analysis correctly identifies the dependence of coupler and follower motions on the Grashof condition and link length ratios.

The project reinforces the utility of computer-aided kinematic analysis in quickly evaluating mechanism performance over complete cycles, which would be tedious to perform manually for every crank angle.

# 6. References

[1]  S. S. Rattan, Theory of Machines, 3rd ed., McGraw-Hill Education, 2009. (Equations 4.12–4.23)

[2]  R. S. Khurmi and J. K. Gupta, Theory of Machines, S. Chand Publishing.

[3]  ME223 Group Project 2 Problem Statement, Department of Mechanical Engineering, 2026.

[4]  Streamlit Documentation — https://docs.streamlit.io